Connecting the Linux Mint Laptop to Home Wi-Fi

Step-by-step setup guide for the MacBook running Linux Mint (Broadcom BCM43602 built-in Wi-Fi)

Before you start. This laptop's built-in Wi-Fi chip is a Broadcom BCM43602, which has two quirks we worked around: it can only reliably see 2.4 GHz networks (not 5 GHz), and it needs its wireless region set to US and management-frame protection (PMF) turned off. These instructions bake in those workarounds so the connection actually holds.

How to use this guide. Lines shown in the shaded boxes are commands you type into a Terminal window, one at a time, pressing Enter after each. To open a Terminal, click the terminal icon in the taskbar (or press Ctrl+Alt+T). Anywhere you see YOUR_..., replace it (including the word) with your real value, keeping the quotation marks.

# Part 1 — Find out what your home network offers

The single most important thing is whether your home router broadcasts a 2.4 GHz network. If it does, this laptop can use it. Check one of these:

• Log into your home router's app or admin page and look at the Wi-Fi settings. You want a network on the 2.4 GHz band. Many home routers broadcast both 2.4 GHz and 5 GHz — sometimes under the same name, sometimes as two separate names (e.g. "MyWiFi" and "MyWiFi-5G").

• If the bands share one name, that is usually fine — the laptop will simply join on 2.4 GHz. If your router lets you give the 2.4 GHz band its own name, that makes things easier and more reliable.

If your home network is 5 GHz only This laptop's built-in chip cannot see 5 GHz. If your home router has no 2.4 GHz band, either enable one in the router settings, or use the USB Wi-Fi adapter (BrosTrend AX9L) — plug it in and it appears as a second wireless interface you can connect through normally. See Part 5.

# Part 2 — Confirm the laptop is ready

Open a Terminal and run this to confirm the wireless region is still set to US:

cat /sys/module/cfg80211/parameters/ieee80211_regdom

It should print US. If it prints 00 instead, skip to Part 4 (Troubleshooting) and set the region before continuing — otherwise the laptop may not see your network.

Next, confirm the Wi-Fi radio is on and the adapter is present:

nmcli radio wifi

Should say enabled. If it says disabled, turn it on with:

nmcli radio wifi on

# Part 3 — Connect to your home network

## Step 1 — Scan for networks

Force a fresh scan, then list what the laptop can see:

sudo nmcli device wifi rescan

Wait about 20 seconds (scanning takes a moment), then:

nmcli device wifi list

Look for your home network's name in the list. Confirm it is on 2.4 GHz by checking its frequency — run this version to see the FREQ column (note: no spaces after the commas):

nmcli -f SSID,FREQ,SIGNAL device wifi list

A frequency around 2412–2472 MHz means 2.4 GHz (good — the laptop can use it). A frequency around 5000+ MHz is 5 GHz, which this built-in chip cannot use.

If your network does not appear Wait a minute and run the rescan again — a newly created network can take a moment to broadcast. If it still will not appear and you are certain it is 2.4 GHz, see Part 4 (Troubleshooting).

## Step 2 — Create the connection profile

Run these commands one at a time. Replace YOUR_HOME_SSID with the exact network name (case-sensitive, keep the quotes) and YOUR_PASSWORD with the Wi-Fi password. The name home is just a label for the profile — you can leave it as-is.

sudo nmcli connection add type wifi con-name home ssid "YOUR_HOME_SSID"

sudo nmcli connection modify home wifi-sec.key-mgmt wpa-psk

sudo nmcli connection modify home wifi-sec.psk "YOUR_PASSWORD"

sudo nmcli connection modify home wifi-sec.pmf disable

sudo nmcli connection modify home 802-11-wireless.band bg

sudo nmcli connection modify home connection.autoconnect yes

Why these extra lines matter wifi-sec.pmf disable turns off protected management frames, which this older Broadcom firmware chokes on — this was the fix for the connection timing out. 802-11-wireless.band bg pins the profile to the 2.4 GHz band. connection.autoconnect yes makes the laptop reconnect on its own after every reboot.

## Step 3 — Bring the connection up

sudo nmcli connection up home

If it works, you will see Connection successfully activated. Open Firefox and load any web page to confirm you are online.

## Step 4 — (Optional) Confirm the password saved

If the connection did not come up, check that the password was actually stored:

sudo nmcli --show-secrets connection show home | grep psk

It should show your real password. If that line is blank, the password did not save — most often because it contains a special character (like $ ! " ' \ or a space) that confused the command. Re-run the wifi-sec.psk line, or temporarily set a simple letters-and-numbers password on the router for testing.

# Part 4 — Troubleshooting

## The network list is empty, or the region shows 00

If cat /sys/module/cfg80211/parameters/ieee80211_regdom shows 00 instead of US, the region reset. This should not happen anymore (a persistence file was created at /etc/modprobe.d/cfg80211-regdom.conf), but if it does, reset the driver with the region set correctly. Run these one at a time:

sudo modprobe -r brcmfmac_wcc

sudo modprobe -r brcmfmac

sudo modprobe -r cfg80211

sudo modprobe cfg80211 ieee80211_regdom=US

sudo modprobe brcmfmac

sudo modprobe brcmfmac_wcc

sudo ip link set wlp2s0 up

If any modprobe -r line says "module in use," first stop NetworkManager with sudo systemctl stop NetworkManager, run the removal lines, then start it again with sudo systemctl start NetworkManager. Then re-check the region and rescan.

Confirm the persistence file exists (so the region survives reboots). This should print options cfg80211 ieee80211_regdom=US:

cat /etc/modprobe.d/cfg80211-regdom.conf

If it says "no such file or directory," recreate it: open the editor with sudo nano /etc/modprobe.d/cfg80211-regdom.conf, type the single line options cfg80211 ieee80211_regdom=US, then save and exit with Ctrl+O, Enter, Ctrl+X.

## It asks for the password over and over, or times out

A repeating password prompt or a 20-second timeout usually is not a wrong password on this chip — it is the association failing. Make sure the profile has wifi-sec.pmf disable set (Part 3, Step 2). Connecting from the Terminal with sudo nmcli connection up home gives clearer errors than the pop-up dialog.

## Cleaning up duplicate profiles

If you end up with several duplicate profiles and one breaks scanning, list them with their IDs:

nmcli -f NAME,UUID connection show

Delete an unwanted one by its UUID (this avoids problems with spaces in names). Deleting a profile is harmless — it does not affect the actual network:

sudo nmcli connection delete PASTE-UUID-HERE

# Part 5 — Using the USB Wi-Fi adapter (BrosTrend AX9L)

When the USB adapter arrives, it gives you fast 5 GHz and sidesteps the built-in chip's limitations entirely. It is plug-and-play — no driver to install.

1. Plug the adapter into the laptop (via your USB-C adapter, or a short USB extension cable so it can sit flat on a table or armrest).

2. Give it a few seconds. Then list your wireless interfaces:

nmcli device status

You should see a new wireless device in addition to wlp2s0 — it will have a name like wlx followed by letters and numbers. That is the adapter.

3. Connect through it to your faster 5 GHz network. You can use the desktop Wi-Fi menu as normal, or the Terminal:

sudo nmcli device wifi connect "YOUR_5GHZ_SSID" password "YOUR_PASSWORD"

With the adapter, you do not need the region reset, the PMF workaround, or the 2.4 GHz restriction — those only applied to the built-in Broadcom chip. The built-in 2.4 GHz connection can stay as a backup.

Quick reference: built-in Wi-Fi needs region US + PMF disabled + 2.4 GHz. USB adapter needs none of that. If Wi-Fi breaks after a reboot, check the region first.